# Clifton Cameras Product Specification

## Sony Alpha A7R II Digital Camera Body Full Spec

Size & Weight

Dimensions (W x H x D)

Approx. 126.9 x 95.7 x 60.3 mm (approx. 5 x 3 7/8 x 2 3/8 inches)

Weight

Approx. 625 g (approx. 1 lb 6.0 oz) (with battery and Memory Stick PRO Duo)

Approx. 582 g (approx. 1 lb 4.5 oz) (body only)

General

Camera type

Interchangeable-lens digital camera

Lens compatibility

E-mount

Image sensor

Type

35 mm full frame (35.9 x 24.0 mm), Exmor R CMOS sensor

Number of pixels (effective)

Approx. 42.4 megapixels

Number of pixels (total)

Approx. 43.6 megapixels

Image sensor aspect ratio

3:2

Anti-dust system

Charge protection coating on optical filter and image sensor shift mechanism

Recording system (still images)

Recording format

JPEG (DCF Ver. 2.0, Exif Ver. 2.3, MPF baseline compliant), RAW (Sony ARW 2.3 format)

Image size (pixels)

[3:2 aspect ratio]

35 mm full frame L: 7,952 x 5304 (42M), M: 5168 x 3448 (18M), S: 3984 x 2656 (11M)

APS-C L: 5168 x 3448 (18M), M: 3984 x 2656 (11M), S: 2592 x 1728 (4.5M)

[16:9 aspect ratio]

35 mm full frame L: 7,952 x 4,472 (36M), M: 5,168 x 2,912 (15M), S: 3,984 x 2,240 (8.9M)

APS-C L: 5168 x 2912 (15M), M: 3984 x 2240 (8.9M), S: 2592 x 1456 (3.8M)

[Sweep Panorama]

Wide: Horizontal 12,416 x 1,856 (23M), vertical 5,536 x 2,160 (12M)

Standard: Horizontal 8,192 x 1,856 (15M), vertical 3,872 x 2,160 (8.4M)

Image quality modes

RAW, RAW & JPEG, JPEG Extra fine, JPEG Fine, JPEG Standard

RAW output

14 bit

Picture Effect

13 modes: Posterization (Color, B/W), Pop Color, Retro Photo, Partial Color (R, G, B, Y), High Contrast Monochrome, Toy Camera, Soft High-key, Soft Focus, HDR Painting, Rich-tone Monochrome, Miniature, Watercolor, Illustration

Creative Style

Standard, Vivid, Neutral, Clear, Deep, Light, Portrait, Landscape, Sunset, Night Scene, Autumn Leaves, Black & White, Sepia (Contrast -3 to +3 steps, Saturation -3 to +3 steps, Sharpness -3 to +3 steps) (Style Box 1-6 also provided)

Dynamic range functions

Off, Dynamic Range Optimizer (Auto/Level (1–5)), Auto High Dynamic Range: Auto Exposure Difference, Exposure Difference Level (1.0-6.0 EV, 1.0 EV step)

Color space

sRGB standard (with sYCC gamut) and Adobe RGB standard compatible with TRILUMINOS™ Color

Recording system (movies)

Recording format

XAVC S / AVCHD format Ver. 2.0 compliant / MP4

Video compression

XAVC S: MPEG-4 AVC/H.264

AVCHD: MPEG-4 AVC/H.264

MP4: MPEG-4 AVC/H.264

Audio recording format

XAVC S: LPCM, 2ch

AVCHD: Dolby Digital (AC-3), 2ch, Dolby Digital Stereo Creator

MP4: MPEG-4 AAC-LC, 2ch

Image size (pixels)

[NTSC]

XAVC S 4K: 3,840 x 2,160 (30p/100 Mbps, 30p/60 Mbps, 24p/100 Mbps, 24p/60 Mbps)

XAVC S HD: 1,920 x 1,080 (60p/50 Mbps, 30p/50 Mbps, 24p/50 Mbps), 1,280 x 720 (120p/50 Mbps)

AVCHD: 1920 x 1080 (60p/28Mbps/PS, 60i/24Mbps/FX, 60i/17Mbps/FH, 24p/24Mbps/FX, 24p/17Mbps/FH)

MP4: 1,920 x 1,080 (60p/28 Mbps, 30p/16 Mbps), 1,280 x 720 (30p/6 Mbps)

[PAL]

XAVC S 4K: 3,840 x 2,160 (25p/100 Mbps, 25p/60 Mbps)

XAVC S HD: 1,920 x 1,080 (50p/50 Mbps, 25p/50 Mbps), 1,280 x 720 (100p/50 Mbps)

AVCHD: 1920 x 1080 (50p/28Mbps/PS, 50i/24Mbps/FX, 50i/17Mbps/FH, 25p/24Mbps/FX, 25p/17Mbps/FH)

MP4: 1,920 x 1,080 (50p/28 Mbps, 25p/16 Mbps), 1,280 x 720 (25p/6 Mbps)

Picture Profile

Yes (Off/PP1-PP7) Parameters: Black level, Gamma (Movie, Still, Cine1-4, ITU709, ITU709 [800%], S-Log2), Black Gamma, Knee, Color Mode, Saturation, Color Phase, Color Depth, Detail, Copy, Reset

Movie functions

Audio Level Display, Audio Rec Level, AF Tracking Sensitivity, AF Drive Speed, Auto Slow Shutter, HDMI info. Display (On/Off selectable), Time Code/User Bit, Picture Profile, Creative Style, Picture Effect, Rec Control, Dual Video Rec, Marker Setting, PAL/NTSC Selector

Color space

xvYCC standard (x.v.Color when connected via HDMI cable) compatible with TRILUMINOS™ Color

Clean HDMI output

NTSC: 3,840 x 2,160 (30p/24p) / 1,920 x 1,080 (60p/24p) / 1,920 x 1,080 (60i), YCbCr 4:2:2 8 bit / RGB 8 bit

PAL: 3,840 x 2,160 (25p) / 1,920 x 1,080 (50p) / 1,920 x 1,080 (50i), YCbCr 4:2:2 8 bit/ RGB 8 bit

Media

Media

Memory Stick PRO Duo, Memory Stick PRO-HG Duo, Memory Stick Micro (M2), SD memory card, SDHC memory card (UHS-I compliant), SDXC memory card (UHS-I compliant), microSD memory card, microSDHC memory card, microSDXC memory card

Slot

Multi slot for Memory Stick Duo / SD memory card

Noise reduction

Noise reduction

Long exposure NR: On/Off, available at shutter speeds longer than 1 sec.

High ISO NR: Normal / Low / Off selectable

Multi Frame NR

Auto/ISO 100 to 102400

White balance

Modes

Auto WB / Daylight / Shade / Cloudy / Incandescent / Fluorescent (Warm White / Cool White / Day White / Daylight) / Flash / Color Temperature (2,500 to 9,900K) & Color Filter (G7 to M7: 57steps, A7 to B7: 29steps) / Custom / Underwater

AWB micro adjustment

G7 to M7 (57 steps), A7 to B7 (29 steps)

Bracketing

3 frames, H/L selectable

Focus system

Type

Fast Hybrid AF (phase-detection AF / contrast-detection AF)

Focus point

35 mm full frame: 399 points (phase-detection AF)

APS-C: 357 points (phase-detection AF) / 25 points (contrast-detection AF)

Sensitivity range

EV -2 to EV 20 (at ISO 100 equivalent with F2.0 lens attached)

AF modes

Automatic AF (AF-A), Single-shot AF (AF-S), Continuous AF (AF-C), Direct Manual Focus (DMF), Manual Focus

Focus area

Wide (399 points for phase-detection AF / 25 points for contrast-detection AF) / Center / Flexible Spot (S/M/L) / Zone / Expand Flexible Spot / Lock-on AF (Wide / Zone / Center / Flexible Spot (S/M/L) / Expand Flexible Spot)

Other features

Lock-on AF, Eye AF, Predictive control, Focus lock; Eye-start AF and AF micro adjustment (both only available with optional LA-EA2 or LA-EA4 attached), AF illuminator (built-in, LED type, range: Approx. 0.30–3 m), AF ON

Exposure control

Metering type

1200-zone evaluative metering

Metering sensor

Exmor R CMOS sensor

Metering sensitivity

EV -3 to EV 20 (at ISO 100 equivalent with F2.0 lens attached)

Metering modes

Multi-segment, Center-weighted, Spot

Exposure modes

AUTO (iAUTO, Superior Auto), Programmed AE (P), Aperture priority (A), Shutter-speed priority (S), Manual (M), Scene Selection, Sweep Panorama, Movie

Scene Selection

Portrait, Landscape, Macro, Sports Action, Sunset, Night Portrait, Night Scene, Hand-held Twilight, Anti Motion Blur

Exposure compensation

+/-5.0 EV (in 1/3 EV or 1/2 EV steps), with exposure compensation dial: +/-3.0 EV (in 1/3 EV steps)

AE bracketing

Bracket: Single/Bracket: Cont., 3/5/9 frames selectable. With 3 or 5 frames, in 1/3, 1/2, 2/3, 1.0, 2.0 or 3.0 EV increments, with 9 frames, in 1/3, 1/2, 2/3 or 1.0 EV increments.

AE lock

Available with AE lock button. Locked when shutter button is pressed halfway. Can be disabled from the Menu.

ISO sensitivity (Recommended Exposure Index)

Still images: ISO 100-25600 (expandable to ISO 50-102400 for shooting still images), AUTO (ISO 100-6400, selectable lower limit and upper limit)

Movies: ISO 100-25600 equivalent, AUTO (ISO 100-6400 equivalent, selectable lower limit and upper limit)

Viewfinder

Type

XGA OLED, 1.3 cm (0.5 type) electronic viewfinder (color)

Total number of dots

2,359,296 dots

Brightness control

Auto/Manual (5 steps between -2 and +2)

Color temperature control

Manual (5 steps)

Field coverage

100%

Magnification

Approx. 0.78x (with 50 mm lens at infinity, -1 m-1)

Diopter adjustment

-4.0 to +3.0 m-1

Eye point

Approx. 23 mm from the eyepiece lens, 18.5 mm from the eyepiece frame at -1 m-1 (CIPA standard)

Display

Graphic Display / Display All Info. / No Disp. Info. / Histogram / Digital Level Gauge

Real-time image-adjustment display

On/Off

LCD screen

Type

7.5 cm (3.0 type) TFT drive

Total number of dots

1,228,800 dots

Brightness control

Manual (5 steps between -2 and +2), Sunny Weather mode

Adjustable Angle

Up approx. 107 degrees, down approx. 41 degrees

Display selector (Finder/LCD)

Auto/Manual

Display

Graphic Display / Display All Info. / No Disp. Info. / Histogram / Digital Level Gauge / Shooting information for viewfinder mode

Real-time image-adjustment display

On/Off

Zebra

Yes

Peaking MF

Yes (Level setting: High/Mid/Low/Off, Color: White/Red/Yellow)

Other features

Face detection

On / On (Regist. Faces) / Off, Face registration, Face selection (Max. number of detectable faces: 8)

Auto Object Framing

Yes

Clear Image Zoom

Still/Movie: Approx. 2x

Digital zoom

Smart zoom (still images) M: Approx. 1.5x, S: Approx. 2.0x

Digital zoom (still images) Approx. 4x

Digital zoom (movies) Approx. 4x

PlayMemories Camera Apps

Yes

Lens compensation

Peripheral shading, chromatic aberration, distortion

Shutter

Type

Electronically controlled, vertical-traverse, focal-plane type

Shutter speed

Still images: 1/8000 to 30 sec., Bulb

Movies: 1/8000 to 1/4 (1/3 step)

NTSC: Up to 1/60 in AUTO mode (up to 1/30 in Auto Slow Shutter mode)

PAL: Up to 1/50 in AUTO mode (up to 1/25 in Auto Slow Shutter mode)

Flash sync. speed

1/250 sec.

Electronic front curtain shutter

Yes, On/Off

Silent Shooting

Yes, On/Off

SteadyShot INSIDE (image stabilization)

Type

Image Sensor-Shift mechanism with 5-axis compensation (Compensation depends on lens specifications)

Compensation effect

4.5 steps (Based on CIPA standard. Pitch/yaw shake only. With Sonnar T* FE 55 mm F1.8 ZA lens mounted. Long exposure NR off)

Flash Control (with optional external flash)

Control

Pre-flash TTL

Flash compensation

+/-3.0 EV (switchable between 1/3 and 1/2 EV steps)

Flash bracketing

3/5/9 frames selectable. With 3 or 5 frames, in 1/3, 1/2, 2/3, 1.0, 2.0, 3.0 EV increments, with 9 frames, in 1/3, 1/2, 2/3, 1.0 EV increments.

Flash modes

Flash off, Autoflash, Fill-flash, Rear Sync., Slow Sync., Red-eye reduction (On/Off selectable), Hi-speed sync*1, Wireless*1

External flash

Sony α System Flash compatible with Multi Interface Shoe. Attach the shoe adaptor for flash compatible with Auto-lock Accessory Shoe.

FE level lock

Yes

Drive

Drive modes

Single shooting, Continuous shooting, Self-timer, Self-timer (Cont.), Bracketing (Cont., Single, White Balance, DRO)

Self-timer

10 sec. delay/5 sec. delay/2 sec. delay/Continuous self-timer (3 frames after 10 sec. delay/5 frames after 10 sec. delay/3 frames after 5 sec. delay/5 frames after 5 sec. delay/3 frames after 2 sec. delay/5 frames after 2 sec. delay)/Bracketing self-timer (Off/2 sec. delay/5 sec. delay/10 sec. delay)

Speed (approx., max.)*2

Continuous shooting Hi: Max. 5fps, Continuous shooting Lo: Max. 2.5fps

No. of recordable frames*2 (approx.)

Continuous shooting Hi: 24 frames (JPEG Extra Fine L), 30 frames (JPEG Fine L), 37 frames (JPEG Standard L), 23 frames (RAW), 22 frames (RAW & JPEG)

Playback

Modes

Single (with or without shooting information, Y RGB histogram & highlight/shadow warning), 9/25-frame index view, Enlarged display mode (Maximum magnification L: 24.9x, M: 16.2x, S: 9.4x), Auto Review (10/5/2 sec, off), Image orientation (Auto/Manual/Off selectable), Slideshow, Panorama scrolling, Folder selection (Still / Date / MP4 / AVCHD / XAVC S HD / XAVC S 4K), Forward/Rewind (Movie), Delete, Protect

Interface

PC interface

Mass-storage, MTP, PC remote

Multi/Micro USB Terminal

Yes*3

Wireless LAN (built-in)

Wi-Fi Compatible, IEEE802.11b/g/n (2.4GHz band)*4

Playback of still images and movies on smartphones, PCs and TVs

NFC™

Yes (NFC Forum Type 3 Tag compatible, One-touch remote, One-touch sharing)

HD output

HDMI micro connector (Type-D), BRAVIA Sync (link menu), PhotoTV HD, 4K still image playback

Multi Interface Shoe

Yes

Others

Auto-lock Accessory Shoe compatible with supplied shoe adaptor, Microphone terminal (3.5 mm Stereo minijack), Headphone terminal (3.5 mm Stereo minijack), Vertical Grip Connector

Audio

Microphone

Built-in stereo microphone or ECM-XYST1M / XLR-K2M (sold separately)

Speaker

Built-in, monaural

Print

Compatible standards

Exif Print, Print Image Matching III, DPOF setting

Custom function

Type

Custom key settings, Programmable setting

Memory function

Yes (2 sets)

Power

Battery

Rechargeable battery pack NP-FW50

Still images

Approx. 290 shots (viewfinder) / approx. 340 shots (LCD screen) (CIPA standard)

Movies

Actual*5*6: Approx. 50 min. with viewfinder, approx. 55 min. with LCD screen (CIPA standard)

Continuous*5*7: Approx. 95 min. with viewfinder, approx. 95 min. with LCD screen (CIPA standard)

External power

AC Adaptor AC-PW20 (optional)

Operating temperature

Range

32–104°F/0–40°C